Date of calibration 03/24/2026 - Room Temperature DI Water - Test #2

Rinse bucket 3 times

Fill bucket with 5 L of water using a 1000 mL graduated cylinder

Launched the sensor using 1 second intervals

Rinse sensor with DI water before each salt addition

Let the sensor sit in the water for 5 minutes to get baseline conductivity before any salt is added.

Measure salt using Sartorius Lab Instruments Practum 124-1S scale

- Salt measured in 0.5 g increments

After taking the baseline conductivity, we took the sensor out and read the data to check if the sensor was reading correctly without stopping the logger. We then rinsed the sensor with DI water and added 0.5 g of salt to the water. We stirred the water thoroughly before adding the sensor back into the bucket. We let the sensor sit for another 5 minutes before taking it out, rinsing the sensor with DI water and adding another 0.5 g of salt to the bucket. Again, we thoroughly stirred the water before adding the sensor back into the bucket. We repeated these steps until 2 g of salt had been added to the water.

Gloves were worn when taking the sensor in and out of the water.

We then read out the data using HOBOware.

| Mass (g) | Water type | Water amount (L) | Cal. Time (min) | Approx. time of day | Date of calibration | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Room temp DI | 5 | 5 | 11:33 – 11:39 | 3/24/2026 |  |
| 0.5105 | Room temp DI | 5 | 5 | 11:42 – 11:47 | 3/24/2026 |  |
| 1.0124 (+ 0.5019) | Room temp DI | 5 | 5 | 11:49 – 11:54 | 3/24/2026 |  |
| 1.515 (+0.5026) | Room temp DI | 5 | 5 | 11:55 – 12:00 | 3/24/2026 |  |
| 2.0165 (+0.5015) | Room temp DI | 5 | 5 | 12:01 – 12:07 | 3/24/2026 |  |